- LG-Nortel Key System

- Technical Information

- STI-0093

- June 01, 2009

| Title | #### IP Ports for iPECS-MG system |
| --- | --- |
| System | iPECS-MG |
| Abstract | This document includes IP ports for iPECS-MG system. |

- Revision History

| Revision | By | History |
| --- | --- | --- |
| 1.0A | S. J. Park | 2009. 06. 01 |

- IP Ports for iPECS-MG system

| Usage | Port | Protocol |
| --- | --- | --- |
| IPKTS Protocol (UNI-CAST) | 5588 | UDP |
| Web Admin | 80 | TCP |
| Telnet (MPB) | 5003 | TCP |
| Telnet (Board) | 5102~5118 (5100 + SLOT NO) | TCP |
| H.323 Signal | 1720 | TCP |
| SIP Signal | 5060 | UDP |
| VOIP RTP Port | 7000 + (SLOTNO*100) ~ 7000 + (SLOTNO*100) + 99 (Example) - Slot 0 (VOIU) : 7000 ~ 7199 - Slot 2 : 7200 ~ 7299 - Slot 3 : 7300 ~ 7399 - Slot 4 : 7400 ~ 7499 - Slot 5 : 7500 ~ 7599 - Slot 6 : 7600 ~ 7699 - Slot 7 : 7700 ~ 7799 - Slot 8 : 7800 ~ 7899 - Slot 9 : 7900 ~ 7999 - Slot 10 : 8000 ~ 8099 - Slot 11 : 8100 ~ 8199 - Slot 12 : 8200 ~ 8299 - Slot 13 : 8300 ~ 8399 - Slot 14 : 8400 ~ 8499 - Slot 15 : 8500 ~ 8599 - Slot 16 : 8600 ~ 8699 - Slot 17 : 8700 ~ 8799 - Slot 18 : 8800 ~ 8899 | UDP |
| PC Application Interface - CTI 3rd Party/ CTI 1st Party - ez Attendant - Phontage | 5588 ~ 5590 | UDP |
| BLF Manager | 9000 9001 | TCP UDP |
| SMDR Interface | 6016 | TCP |
| SNMP Agent | 161 | UDP |
| NMS Server | 162 | UDP |