LG PBX System

### Software Technical Information

STI-0077

August 28, 2001

| Title | #### VPM Temporary User Defined Ment Use |
| --- | --- |
| System | CS1000 |
| Abstract | Function to use a temporary announcement for a certain set time, instead of CCR User Defined Announcement or Voice Tone used for IVR service |

Revision History

| Revision | Date | By | History |
| --- | --- | --- | --- |
| 1.0 | 2001.08.24 | H.S Kim | Initial Development |
| 1.1 | 2001.09.18 | H.G Jung | Voice Tone supporting function that is added |

Table of Contents

Software Technical Information	1

Title: VPM Temporary User Defined Announcement	2

Functional Description	2

Operational Procedure	2

Conditions	3

Hardware	3

Data Generation	3

Interaction	3

OMS Menu	3

Title: VPM Temporary User Defined Announcement

Functional Description

This function is to use a temporary announcement for a certain set time, instead of specified announcement like CCR User Defined Announcement or Voice Tone used for IVR service, when running VPM for the service as below.

- User Defined Announcements set in IVR Service per CCR Cable

- Announcement in Voice Tone service per ID

Operational Procedure

- [Temporary Announcement Recording]

- Put a telephone into Off Hook state. (SLT or DKTU).

- Dial Calling Group Representative Number for Token Change that is specified.

- Record token for Announcement following VPM voice information.

- [Registration Temporary Announcement for IVR Service]

- Use DKTU to enter a Code(Default : *997) corresponding to the function The displays will be shown in order as below.

- CCR:## UDM:# EM:#

- Start ####/##/## ##.##

- End ####/##/## ##.##

- Enter CCR Table Number(Range : 01 ~ 40) following LCD Display Indication.

- Enter User Defined Announcement Index No(Range : 01 ~ 03) following LCD Display Indication.

- Enter Token Index(Range : 0 ~ 9) for Temporary Announcement following LCD Display Indication.

- Enter Start Time following LCD Display Indication.

- Enter End Time following LCD Display Indication.

- Hear Function end tone.

- Put the phone into on-hook state.

- [Registration Temporary Announcement for Voice Tone Service]

- Use DKTU to enter a Code(Default : *997) corresponding to the function The displays will be shown in order as below.

- CCR:## UDM:# EM:#

- Start ####/##/## ##.##

- End ####/##/## ##.##

- Enter Voice Tone Service Identification No. 99 following LCD Display Indication.

- Enter Voice Tone ID No(Range :01 ~ 50) following LCD Display Indication.

- Enter Token Index(Range : 0 ~ 9) for Temporary Announcement following LCD Display Indication.

- Enter Start Time following LCD Display Indication.

- Enter End Time following LCD Display Indication.

- Hear Function end tone.

- Put the phone into on-hook state.

- [Verify & Cancellation Temporary Announcement]

- Use DKTU to enter Code(Default : *998) corresponding to the function.

- Use ‘Arrow Key’ to check information recorded as below.

- Ex) In case you substitute Temporary Announcement Token 5 for User Defined Announce 2 in CCR Table 1 from 2001/09/10 09:00 to 2001/09/11 18:00

- 01 2 5 10-09:00 11-18:00

- Ex) In case you substitute Temporary Announcement Token 5 for Voice Tone ID 7 from 2001/09/10 09:00 to 2001/09/11 18:00

- 99 7 6 10-09:00 11-18:00

- Press the DND button when Temporary Announcement items you want to delete is displayed.

Conditions

- The max number of Temporary Announcement Tokens, which are usable simultaneously, is 10.

- Only user(s) specified in the system can record Temporary Announcement over the phone.

- Temporary Announcement that has been registered by recording will be automatically released past a set time, unless otherwise cancelled.

- Each of Temporary Announcement and Bulletin Service uses the same resources in a VPM so that both cannot be used together.

- Information on registration will be saved to Func.dt so that the service can be maintained even in case of CPM restarting.

Hardware

N/A

Data Generation

  - Choosing user to record Temporary Announcement Token

- [3.1 Extension Information -> Extension Characteristic, 13.Token Change item]

  - Equivalent CCR table service decision after playing Temporary Announcement.

- [15.5 Voice mail Option -> VPM Bulletin Service Use?]

- Bulletin Service=Yes: Equivalent CCR table service is executed after playing temporary announcement instead of

- CCR table announcement.

- Bulletin Service=No: Night service of equivalent CCR table is executed after playing temporary announcement

- instead of CCR table announcement.

Interaction

This service is available only for CPM.IMG 2.01 version or greater and VPM 2.01 version or greater.

OMS Menu

N/A