# LG-Ericsson Key System

# Technical Information

# STI-0096

# February 10, 2012

| Title | iPECS-MG Troubleshooting Guide – Mailbox full in 1.7Cd |
| --- | --- |
| System | iPECS-MG |
| Abstract | This document is guide for Mailbox Full Problem in 1.7Cd |

# Revision History

| Revision | By | History |
| --- | --- | --- |
| 1.0A | S. J. Park | 2012. 02. 10 |

# Troubleshooting

# 1. PROBLEM/SYMPTOM

When a user installs new iPECS-MG system (version 1.7Cd), LCD on DKT/LIP phone can be displayed “MAILBOX IS FULL”.

# 2. CAUSE

DB Protection Option (PGM 223-21) can be set to VM DB when a memory (SRAM) is discharged.

If this is set to “VM DB”, iPECS-MG system does not initialize DB related to Voice Mail.

# 3. SOLUTION

A user can disable DB Protection Option in Admin Program.

- Set DB Protection Option to OFF in PGM 223-21

- Set the Position of SW1-1 on MPB to OFF

- Set the Position of SW1-2 on MPB to ON

- Reset the system

- DB Compatibility

- MPB S/W Upgrade

When MPB S/W is upgraded from under V1.7 to V2.0, all of previous data will be kept.

User can use previous functions. All of features work well.

- MPB S/W Downgrade

In case of Downgrade from V2.0 to under V1.7 S/W, data of DB has to be initialized.

- V1.7 DB file Upload to V2.0 MPB

Saved V1.7 DB file can upload to V2.0 system

- V2.0 DB file Upload to V1.7 MPB